M960 Screen Tester

Product Manual

V1.0

Shenzhen Yuchengxing Electronics Technology Co., Ltd.

Address: 3F, Building 4, Rihai Intelligent Technology Park, Longhua District, Shenzhen, China

Service Hotline: +86-755-23221950

Note: Images are for illustrative purposes only. Actual product appearance may vary.

Table of Contents

1.1Product Advantages....................................................4

1.2Main Function Menu Description...............................5

2.1Button Functions.........................................................7

2.2Flex Cable Connection Diagram.................................8

3.M960 Tester Power On/Off............................................9

4.Screen Display Testing..................................................11

5.Touch Line Drawing Test..............................................13

6.Brightness Cycle............................................................14

7.True Color Restoration..................................................15

8.APP Upgrade..................................................................16

9.Product Specifications & Packaging........................... 18

10.Warranty & Non-Warranty Terms...............................19

11.Usage Precautions......................................................20

  - 1.1 Product Advantages：

  - 1.Multi-Interface Compatibility

  - Supports eDP (ALP-DP), MIPI, and QSPI interfaces, enabling full-speed eDP signal display output.

- 2.Universal Screen Compatibility

- Compatible with displayand touch testing for OLED, TFT, Original (OEM), Aftermarket, and Compressed screens.

- 3.Real-Time Power Monitoring

- Built-in voltage/current acquisition module for real-time screen power consumption tracking.

- 4.Multi-Layer Circuit Protection

- Integrated multi-stage over-current (OCP) and over-voltage (OVP) protection within the test box.

- 5.Custom Display Configuration

- Supports user-defined screen content (static images/dynamic videos).

- 6.Apple True Tone Restoration

- Enables True Tone display calibration and color temperature recovery for Apple devices.

- 7.Firmware Upgradable via App

- Supports wireless OTA (Over-The-Air) updates for test protocols and software versions.

  - 8.Brightness Cycle Testing

  - 9.Refresh Rate Verification

  - Tests dynamic refresh rates (e.g., 60Hz/90Hz/120Hz) and detects frame rate jitter.

  - 10.LCD Driver IC Identification

  - Automatically reads LCD driver IC models.

  - 11.The screen test models are complete and the update speed is fast.

  - 1.2Product Main Function Menu Description：

  - 2.1Button Functions：

- K1 Button

- Short press when powered: Cycles through brightness levels.

- Long press (3 seconds) in any state: Initiates model recommendation and filter testing.

- K2 Button

- Long press (3 seconds) in standby or powered state: Switches between LCD+TP, LCD-only, or refresh rate modes.

- K3 Button

- Long press (3 seconds) in standby or powered state: Switches between HAND (Manual) and AUTO (Automatic) modes.

- Up Button

- Short press in standby: Powers on the device.

- Short press when powered: Cycles backward through display images.

- Down Button

- Short press in standby: Powers on the device.

  - Auto Shutdown

  - Powers off automatically after 10 minutes of inactivity in standby mode.

  - 2.2 Schematic diagram of cable connection：

- 3 M960 test box on/off

- Power on: Plug in the DC power cord and switch the "Power Switch" to the "ON" state to turn on the power of the M960 test box.

- After the built-in screen of the test box lights up and works normally, it can be successfully turned on.

- Shutdown: Switch the "Power Switch" to the "OFF" state to turn off the power of the M960 test box. Simply unplug the power cord.

- 4 Screen Display Testing Procedure

- Step 1: Preparation before testing: The "test screen" corresponds to the adapter board and adapter cable (eDP adapter board needs to be connected to the cable with eDP letters written on it)

- Step 2: Close the screen, click the power button, and click "Flip Up, Flip Down" to switch to the test screen and view the effect.

- Step 3: Turn off the screen, click on the "power button" to turn off the screen.

- Step 4: Repeat the above steps to continue testing the next screen

- Note: Switching program: When the screen is not powered on, click the up button icon on the small screen to switch

- 5  Touch Line Drawing Test

- Step 1: After the "test screen" lights up, click "flip up, flip down" to switch to the touch line test interface normally

- Step 2: Finger marking test screen function

- 6 Brightness Cycle Function

- Step 1: Tap the Brightness Cycle icon in the top-right corner of the screen or press the K1 button to activate brightness cycling.

- 7 Screen True Color Restoration

- Step 1: Hardware Connection

- Mount the screen to the corresponding adapter board and flex cable.

- After the system recognizes the connected device, press the Back button to return to the Main Menu.

- Step 2: Initiate True Color Restoration

- Tap the True Color Restoration icon on the interface.

- Step 3: Select Read Mode

- Read Mode Options:

- LCD: Direct read via display interface.

- I2C: Read through I²C protocol.

- PC: Requires connection to a host computer (software-controlled).

- Switch Modes: Use the Up/Down buttons to toggle between modes.

- Execute Read Operation:

- Mount the source screen (the screen containing original color data).

- Click the Read button.

- Upon success, the message "Read Original Color OK!" will appear at the bottom of the screen.

- Step 4: Write Original Color Data

- Prepare Target Screen:

- Remove the source screen and mount the target screen (the screen to be calibrated).

- Click the Write button.

- Completion Confirmation:

- A success message "Write Original Color OK!" will display at the screen bottom.

- 8 APP Online Update Function

Yuchengxing Assistant APP Online Upgrade: Get Installation Package

1. Log in to the Chinese website of Yuchengxing and download: http://www.ycxbest.com/M960.html

English website：www.ycxbest.com/en/M960.html

- Update Procedure

- After installation, log in to the APP.

- Power on the device and connect it to a computer to initiate the firmware update.

- Note: For detailed instructions, download the installation manual and video guides.

- 9 Product Specifications & Packaging

- Product Parameters：

- Model: M960

- Material: Aluminum Alloy

- Color: Dark Gray

- Power Supply: DC12V—1.5A

- Power Interface: DC interface

- Operating Temperature: 25℃ to 85℃

- Operating Humidity: <90% RH

- Dimensions: 170mm × 90.8mm × 21.7mm

- M960 bare machine weight: 285g ± 10g

- Storage Conditions: -30℃ to 85℃, <90% RH

- Package Contents：M960 Screen Tester、 DC charging cable

- Packaging Details

- 10 Warranty & Non-Warranty Terms

- Warranty Coverage：

- 7-Day Policy

- Within 7 days from the date of receipt, if the product exhibits performance failures confirmed by our after-sales service center, you are entitled to free return, replacement, or repair.

- 2.8-15 Day Policy

- Between 8-15 days from the date of receipt, confirmed performance failures qualify for free replacement or repair.

- 3.1-Year Limited Warranty

- Within 1 year from the date of receipt, confirmed performance failures qualify for free repair services.

- Non-Warranty Cases:                                                       4.1. Damage caused by unauthorized disassembly, repair, misuse, modification, or improper storage.                                                                   4.2.Use of non-original accessories or tampering with product labels (e.g., tearing, altering).

- 4.3.Products exceeding the warranty period.

- 4.4.Damage due to natural disasters or force majeure.

- 4.5. Damage caused by using non-compliant chargers (exceeding specified voltage ratings).

- 5.*Final Interpretation Rights

- Yuchengxing Electronics Technology Co., Ltd. reserves the right to interpret these terms within legal limits.

- 11 Usage Precautions

- Power Supply

- Ensure the power supply voltage matches the device's rated voltage before connecting to the testing box.

- Use original manufacturer-provided charging cables.

- Additional Guidelines：

- 1.Environmental Conditions

- Avoid using the device in high-temperature or humid environments to prevent reduced lifespan or damage.

- 2.Interface Compliance

- Input/output ports are clearly labeled. Ensure correct connections during operation.

- 3.Tampering Prohibition

- Do not disassemble the device, as this may cause damage or compromise testing accuracy.

- 4.Maintenance

- Regularly clean accumulated dust and debris to maintain device hygiene.

- 5.Mechanical Stress

- Protect the device from mechanical shocks or impacts.